# Setup B Spoon Effective Sample Size Report

Setup: setup_b

Report timestamp: 20260420_095917

Source notebook: setup_B_spoon_effective_sample_size.ipynb

# Executive summary

Setup B is treated as a deterministic sequence of spoon presentations during one spectral acquisition. The report separates the static optical mass sampled by one spoon from the number of spoon presentations that occur during the acquisition window, then combines them into an effective sampled mass per spectrum. The dominant trend is that shorter spoon intervals increase the number of contributing presentations and therefore increase effective sampled mass.

# Key parameters

Probe diameter (mm): 5

Penetration depth range (mm): 0.1 to 2.0

Penetration depth step (mm): 0.1

Bulk densities (g/cm^3): 0.4, 0.5, 0.6

Subscans: 256

Integration time (ms): 15

Acquisition time (s): 3.84

Spoon intervals (s): 1, 2, 3, 4, 5

## Background

Setup B represents a spoon or flip-sampler interface where powder originates from a flowing process stream, is captured in a spoon, and is measured by the NIR probe while static during each spoon presentation.

The upstream process is dynamic, but each optical event is treated as a static-bed measurement during the spoon exposure. During one reported spectrum, multiple spoon presentations may occur. Under the assumptions used here, effective sample size is governed by optical interaction mass per spoon, total acquisition time, and spoon flipping frequency. It is not directly governed by upstream throughput in this first deterministic model.

## Objective

This notebook simulates Setup B effective sample size by quantifying:

static optical sampled mass per spoon

number of spoon samples contributing during one spectrum

effective sampled mass per spectrum

as functions of penetration depth, bulk density, flip frequency, and acquisition settings.

## Theory

For one spoon presentation (static measurement):

Acquisition timing:

Effective sampled mass during one reported spectrum:

Symbol definitions and units:

D_probe: probe diameter [m]

A_probe: probe area [m^2]

d_pen: effective penetration depth [m]

rho_bulk: bulk density [kg/m^3]

V_static: static sampled volume per spoon [m^3]

m_static: static sampled mass per spoon [kg]

N_subscans: number of averaged sub-scans [-]

t_int: integration time per sub-scan [s]

t_acq: total acquisition time [s]

f_flip: spoon flipping frequency [Hz]

t_spoon: spoon interval [s]

N_spoons: spoon samples contributing during one acquisition [-]

m_eff: effective sampled mass per spectrum [kg]

## Interpretation of the Model

In this setup, one spoon presentation is one independent static optical sample. If acquisition time spans multiple spoon presentations, the reported spectrum integrates multiple independent increments.

In this notebook, N_spoons is allowed to be non-integer as a deterministic first approximation for effective mass scaling. Under the stated assumptions, throughput is not explicitly modeled.

## Assumptions

spoon always completely filled

filling is reproducible

consecutive spoons are independent

powder is static during each spoon measurement

circular probe window

cylindrical optical interaction volume approximation

uniform bulk density

no resampling within a spoon

no segregation correction

no stochastic independence correction

no detailed radiative transfer model yet

# Worked example

A representative deterministic spoon-presentation case is shown for density 0.4 g/cm^3, penetration depth 1.0 mm, and spoon interval 3.0 s. This makes the multiplicative logic of the model explicit: static sampled mass per spoon times the number of spoon presentations within one acquisition window.

| Quantity | Value | Unit |
| --- | --- | --- |
| Bulk density | 0.4 | g/cm^3 |
| Penetration depth | 1.0 | mm |
| Spoon interval | 3.0 | s |
| Static sampled mass per spoon | 7.8540 | mg |
| Spoons contributing | 1.2800 | - |
| Effective sampled mass per spectrum | 10.0531 | mg |

# Model scenarios and sweep design

The Setup B sweep combines a static optical model for one spoon with a deterministic presentation count over the acquisition window.

penetration depth from 0.1 to 2.0 mm in 0.1 mm steps

bulk-density scenarios of 0.4, 0.5, 0.6 g/cm^3

spoon intervals of 1, 2, 3, 4, 5 s

acquisition time fixed by 256 subscans at 15.0 ms integration time

This produces 300 modeled scenarios.

# Selected results table

The selected table below focuses on the representative density of 0.4 g/cm^3 and shows how penetration depth and spoon interval jointly affect the final effective sampled mass.

| penetrationdepthmm | spoonintervals | spoons_contributing | staticsampledmass_mg | effectivesampledmass_mg |
| --- | --- | --- | --- | --- |
| 0.5 | 1.0 | 3.8400 | 3.9270 | 15.0796 |
| 0.5 | 3.0 | 1.2800 | 3.9270 | 5.0265 |
| 0.5 | 5.0 | 0.7680 | 3.9270 | 3.0159 |
| 1.0 | 1.0 | 3.8400 | 7.8540 | 30.1593 |
| 1.0 | 3.0 | 1.2800 | 7.8540 | 10.0531 |
| 1.0 | 5.0 | 0.7680 | 7.8540 | 6.0319 |
| 2.0 | 1.0 | 3.8400 | 15.7080 | 60.3186 |
| 2.0 | 3.0 | 1.2800 | 15.7080 | 20.1062 |
| 2.0 | 5.0 | 0.7680 | 15.7080 | 12.0637 |

# Figures

The figures are ordered from static optical geometry to deterministic presentation count to the final effective sampled-mass response.

Figure 1. Static sampled volume per spoon versus penetration depth.

Figure 2. Static sampled mass per spoon for representative bulk-density states at 1.0 mm penetration depth.

Figure 3. Deterministic number of spoon presentations contributing during one acquisition.

Figure 4. Effective sampled mass versus time between spoon flips for the representative density and depth case.

Figure 5. Effective sampled mass versus spoon interval for selected penetration depths at 0.4 g/cm^3.

## Results Interpretation

Plot 2 shows static sample mass per spoon for three representative bulk densities: 0.4, 0.5, and 0.6 g/cm^3 at a penetration depth of 1.0 mm

Plot 4 uses only representative bulk density 0.5 g/cm^3

Plot 5 uses representative bulk density 0.4 g/cm^3

Plot 4 x-axis is time between spoon flips (s), not flip frequency (Hz)

shorter spoon intervals produce more independent spoon presentations during acquisition, increasing effective sample size

## Limitations

independence between spoons is assumed

complete and reproducible filling is assumed

no explicit throughput effect is included

no dynamic filling physics is modeled

no partial spoon overlap model is included

no integer/discrete spoon capture model is enforced

no optical weighting correction yet

# Appendices

## Next logical extensions

enforce integer spoon counting or partial spoon weighting

include incomplete filling scenarios

include non-independent spoons

include spoon geometry explicitly

include particle count estimation

include uncertainty in penetration depth and bulk density

# Exported artifacts

CSV files

results_full.csv

summary_table.csv

input_summary.csv

flip_summary.csv

plot2_static_sample_mass_table.csv

plot4_spoon_interval_effect_table.csv

selected_depths_interval_table.csv

Figure files

plot_01_static_sampled_volume_per_spoon.png

plot_02_static_sample_mass_per_spoon.png

plot_03_spoons_contributing_vs_interval.png

plot_04_effective_sample_size_vs_flip_interval.png

plot_05_effective_mass_vs_flip_interval_selected_depths.png