# Setup E Effective Sample Size Report — PAT Rig

# Run Metadata

Setup name: setup_e

Run timestamp: 20260317_124743

Notebook filename: setup_E_PATrig_effective_sample_size.ipynb

Run directory: C:\temp files\EFS\reports\setup_e\20260317_124743

# Notebook Narrative

# Setup E - Effective Sample Size for PAT Rig NIR Measurement

Powder plug-flow through a pipe with a flush-mounted NIR probe, deterministic first-generation model

## Background

Setup E models a PAT (Process Analytical Technology) rig scenario where powder flows through a pipe past a flush-mounted diffuse-reflectance NIR probe. Unlike the static Setup A, powder motion during each spectral acquisition creates a dynamic sampling situation.

Three distinct mass metrics are relevant:

Instantaneous probed mass (m_inst): mass simultaneously illuminated at any single instant.

New mass presented per acquisition (m_new): unique material refreshed across the probe window during one acquisition time, bounded by the instantaneous mass.

Mass passed probe footprint (m_pass): total mass that crosses the probe window plane during acquisition — grows linearly with time and is not bounded by instantaneous mass.

This notebook uses plug-flow assumptions to compute these metrics and sweeps key operating parameters (acquisition time, throughput) to examine their sensitivity.

## Objective

Compute instantaneous probed mass, new mass presented per acquisition, and mass passed the probe footprint for a PAT rig configuration. Sweep acquisition time and throughput to examine sensitivity, and export results using the standard reporting workflow.

## Theory

Powder velocity from throughput and pipe cross-section:

Instantaneous probed mass (mass in view at any instant):

Axial advance during acquisition:

Refresh fraction (fraction of probe volume replaced during acquisition):

New mass presented per acquisition:

Mass passed the probe footprint during acquisition:

Symbols and units:

dotm: mass flow rate [g/s]

rho_bulk: bulk density [g/cm^3]

A_pipe: pipe cross-section area [cm^2]

A_window: probe window area [cm^2]

d_pen: measurement (penetration) depth [cm]

T_acq: spectral acquisition time [s]

L_axial: axial sampling length (proxy for probe window diameter) [cm]

## Assumptions

plug-flow velocity profile (uniform across pipe cross-section)

uniform bulk density

circular pipe and circular probe window flush-mounted on the pipe wall

measurement depth is small relative to pipe diameter

axial sampling length approximated by probe window diameter

no turbulence, no particle segregation

no detailed radiative transfer model yet

## Input Parameters

Baseline PAT rig configuration:

pipe diameter: 10.0 cm

probe window diameter: 0.5 cm (5 mm)

measurement depth: 0.1 cm (1 mm)

bulk density: 0.3 g/cm^3

throughput: 95 kg over 60 min

acquisition time: 1.0 s

axial sampling length model: window diameter

Sweeps:

acquisition time: 0.1 s to 5.0 s (20 points)

2D sweep: throughput 10–90 kg × acquisition time 0.1–5.0 s

Internal calculations use CGS units.

## Compute Effective Sample Size Metrics

Run 1D acquisition-time sweep and 2D throughput × acquisition-time sweep.

## Interpretation of the Simulation Table

Each row corresponds to one combination of operating parameters. Key columns:

probemassmg_instant: mass simultaneously illuminated at any instant

newmassmgperacq: unique material refreshed during one acquisition (plateaus at instantaneous mass)

masspassedmgperacq: total mass crossing the probe window plane during acquisition (grows linearly with time)

refreshfractionper_acq: fraction of probe volume replaced during acquisition (saturates at 1.0)

effectivesamplesize_mg: treated as new mass presented per acquisition

## Plot 1: New Mass per Acquisition vs Acquisition Time

Shows how the unique mass contributing to a single spectrum changes with acquisition time. At short acquisition times the probe volume is only partially refreshed; at longer times the refresh fraction saturates at 1.0 and new mass plateaus.

## Plot 2: Mass Passed Probe Footprint vs Acquisition Time

Unlike new mass, the mass that passes the probe footprint grows linearly with acquisition time and is not bounded by the instantaneous mass.

## Plot 3: Refresh Fraction vs Acquisition Time

The refresh fraction indicates what fraction of the probe volume is replaced during one acquisition. It saturates at 1.0 when the powder advances at least one probe-window diameter.

## Plot 4: Heatmap — Mass Passed Probe Footprint (Throughput vs Acquisition Time)

2D sensitivity map showing how mass passed the probe footprint depends jointly on throughput and acquisition time.

## Plot 5: Heatmap — New Mass per Acquisition (Throughput vs Acquisition Time)

2D sensitivity map for the refresh-based unique-mass metric.

## Results Interpretation

new mass per acquisition saturates once the powder advance exceeds the probe window diameter (full refresh)

mass passed the probe footprint grows linearly with both acquisition time and throughput

at low throughput or short acquisition times, refresh fraction is small and the effective sample size is limited

the heatmaps reveal the operating envelope where effective sampling is most sensitive to parameter changes

## Practical Implications

acquisition time selection trades off spectral quality against how much unique material is sampled

throughput directly controls powder velocity and therefore refresh rate

for real-time PAT decisions, understanding whether the probe has seen enough unique material is critical

## Limitations

plug-flow assumption ignores radial velocity profile

no turbulence or mixing model

penetration depth is an effective parameter, not a sharp boundary

bulk density assumed uniform and constant

no particle-level sampling statistics

no correction factor for optical weighting yet

## Next Logical Extensions

radial velocity profile (laminar / turbulent)

particle count estimation from effective sample size

sensitivity analysis for measurement depth

uncertainty bands for penetration depth and bulk density

non-uniform optical weighting functions

## Export and Reporting

This export section writes timestamped CSV, PNG, JSON, and Word outputs into the setup-specific reports/ folder. python-docx and nbformat are required for Word export with Markdown rendering.

# Purpose

Setup E estimates the effective sample size for a PAT rig configuration where powder flows through a pipe past a flush-mounted NIR probe. It sweeps acquisition time and throughput to examine sensitivity of instantaneous mass, new mass per acquisition, and mass passed the probe footprint.

# Assumptions

Plug-flow velocity profile, uniform bulk density, circular pipe and probe window, measurement depth small relative to pipe diameter, axial sampling length approximated by probe window diameter, no turbulence or mixing.

# Parameter Summary

Pipe diameter (cm): 10

Probe window diameter (cm): 0.5

Measurement depth (cm): 0.1

Bulk density (g/cm^3): 0.3

Throughput (kg): 95

Throughput time (s): 3600

Acquisition time range (s): 0.1 to 5.0

Axial length model: window_diameter

# Exported CSV Files

results_full.csv

summary_table.csv

input_summary.csv

Detailed tables are saved separately as CSV files in the run folder.

# Figures

plot_01_new_mass_per_acq_vs_acquisition_time.png

plot_02_mass_passed_vs_acquisition_time.png

plot_03_refresh_fraction_vs_acquisition_time.png

plot_04_heatmap_mass_passed_throughput_vs_acquisition.png

plot_05_heatmap_new_mass_throughput_vs_acquisition.png

# Closing Note

This report captures the notebook narrative, summary metadata, and exported artifacts for this run.