# Setup C Effective Sample Size Report

# Run Metadata

Setup name: setup_c

Run timestamp: 20260318_132012

Notebook filename: setup_C_spiderwheel_effective_sample_size.ipynb

Run directory: C:\temp files\EFS\reports\setup_c\20260318_132012

# Notebook Narrative

# Setup C -- Spiderwheel-Based Effective Sampled Mass Estimate

First-order mechanistic approximation for wheel-driven powder renewal and binary probe blocking during one NIR spectral acquisition.

## Background

Setup C represents a spiderwheel-driven powder presentation geometry for NIR diffuse reflectance measurement. As in the other notebooks in this series, the interaction region is treated as a shallow near-surface region in front of the probe window, and the sampled mass is approximated from probe footprint, assumed penetration depth, and bulk density.

The key physical difference from Setup B is that Setup B treats each spoon presentation as a static measurement event, whereas Setup C uses wheel motion to actively renew powder at the probe location during a single acquisition. The spiderwheel therefore introduces two competing effects: it can increase the rate at which newly presented powder passes through the probe region, but the fingers can also intermittently block the probe and reduce the usable observation time.

This notebook is therefore positioned as a first-order mechanistic approximation and a deterministic renewal model. It is intended to quantify effective sampled mass trends and blocking trends, not to claim a validated statistical effective sample size or a full radiative-transfer description of the measurement.

## Objective

This notebook estimates, for Setup C:

sampled mass per refresh event

blocked and unblocked acquisition time during one spectrum

number of refresh opportunities during one spectrum

effective sampled mass estimate per spectrum

sensitivity of the estimate to penetration depth, bulk density, and wheel RPM

## Theory

Wheel kinematics at the probe position:

First-order footprint and sampled mass per refresh event:

Characteristic renewal time scale:

Effective unblocked acquisition time:

Deterministic renewal count:

Effective sampled mass estimate during one acquisition:

In the current implementation, blocked time is estimated from a simple overlap-angle approximation in which finger width and probe width are converted into an angular overlap at the probe radius. This produces a deterministic blocked-time estimate for the acquisition window under the binary blocking approximation.

Symbol definitions and units:

RPM: wheel rotation rate [rev min^-1]

omega: angular speed [rad s^-1]

r_probe: probe radial position from wheel center [cm]

v: tangential speed at the probe radius [cm s^-1]

D_probe: probe window diameter [cm] in calculations, reported as [mm] in engineering tables and labels

A_probe: probe footprint area [cm^2]

d_pen: assumed penetration depth [cm] in calculations, reported as [mm] in engineering tables and labels

V_sample: sampled volume per refresh event [cm^3]

rho_bulk: bulk density [g cm^-3]

m_sample: sampled mass per refresh event [g]

t_acq: total spectral acquisition time [s]

t_blocked: time during which the probe is treated as blocked by a spiderwheel finger [s]

t_eff: effective unblocked acquisition time [s]

t_cross: characteristic crossing time for one probe diameter of tangential motion [s]

N_refresh: deterministic renewal count during one acquisition [-]

m_eff,est: effective sampled mass estimate during one acquisition [g]

The code uses mixed practical engineering units for convenience: geometry is entered mainly in cm and mm, density in g/cm^3, time in s, and reported masses are often shown in mg. The calculations remain internally consistent because the unit conversions are handled explicitly in the code.

## Interpretation of the Model

In this notebook, a refresh means the passage of newly presented powder across the probe region due to wheel motion. It is best interpreted as a renewal opportunity for material presented to the probe window, not as proof of a statistically independent new sample.

Tangential speed is evaluated at the probe radius because the local renewal rate should be tied to the wheel speed where the probe actually views the material. The crossing time t_cross is then interpreted as a characteristic time scale for a wheel-driven material segment to traverse one probe width.

Blocked time is subtracted because the model uses a binary blocking approximation: when a spiderwheel finger overlaps the probe region, the probe is treated as having no usable access to powder during that interval. The resulting t_eff is therefore an effective observation time rather than the full instrument acquisition time.

Nrefresh should therefore be interpreted as a deterministic renewal count, rather than a demonstrated count of independent samples. Powder recirculation, revisit probability, local residence-time memory, and correlation between successive refresh events are not modeled. For the same reason, meff,est is an effective sampled mass estimate, not yet a validated statistical effective sample size and not a direct estimate of analytical variance.

Despite these simplifications, the model is still useful as a mechanistic engineering tool because it shows how geometry, density, penetration depth, RPM, and blocking assumptions influence the amount of powder that is plausibly renewed in front of the probe during one spectrum.

## Assumptions

circular probe window with area represented by A_probe

first-order cylindrical sampled volume approximation, Vsample = Aprobe d_pen

constant bulk density over the sampled region

constant assumed penetration depth for each simulation condition

local renewal rate represented by tangential velocity at the probe radius

binary blocking behavior when a spiderwheel finger overlaps the probe region

no partial transmission, partial attenuation, or partial exposure through a finger

no revisit probability or recirculation correction for previously presented powder

no depth-wise weighting within the assumed sampling volume

no stochastic treatment of particle independence or correlation between refresh events

no explicit radiative-transfer solution

## Input Parameters

The baseline geometric, optical, and operating parameters are defined below. Engineering units are retained in variable names, plot labels, and summary tables for readability, while the calculations remain internally consistent in centimeter-gram-second style units.

## Imports

## Helper Functions

## Single-Case Worked Example

The following worked example evaluates one representative operating condition so that the intermediate quantities can be read directly before the full parameter sweep is run. The sequence is intentionally mechanistic: first the sampled mass per refresh event is estimated, then blocking is estimated, then the effective unblocked acquisition time is obtained, and only then is the effective sampled mass estimate calculated through the deterministic renewal count.

## Parameter Sweep / Simulation

The simulation sweeps bulk density, wheel RPM, and penetration depth while holding the remaining geometry fixed. Each row in the output table represents one deterministic operating condition for the current first-order model.

## Interpretation of Simulation Table

Each row combines one density, one RPM, and one penetration depth. The most important columns are:

sampledmassperrefreshmg: the first-order sampled mass associated with one renewal opportunity at the probe

blockedtimes: the portion of the acquisition treated as unavailable because a spiderwheel finger blocks the probe region

n_refresh: the deterministic renewal count during the unblocked part of the acquisition

effectivesampledmassestimatemg: the effective sampled mass estimate accumulated over one spectrum

This table should therefore be read as a mechanistic comparison table. It does not demonstrate that the listed renewal opportunities are statistically independent, and it does not by itself establish a validated independent effective sample mass.

## Plot 1: Effective Sampled Mass Estimate vs Penetration Depth

This plot isolates how the effective sampled mass estimate changes with penetration depth at a representative wheel speed of 20 RPM. In the present first-order volume approximation, penetration depth scales the sampled volume and therefore the sampled mass per refresh event linearly for fixed density.

Comparing the density curves shows how increased bulk density multiplies that per-refresh sampled mass contribution without changing the blocking geometry itself.

## Plot 2: Effective Sampled Mass Estimate vs RPM

This plot isolates the operating-speed effect at a representative penetration depth of 1.0 mm. Increasing RPM increases tangential speed at the probe radius and therefore decreases the characteristic crossing time, which tends to increase the deterministic number of renewal opportunities.

That increase should not be interpreted as a direct increase in independent sample count. The plotted result is a first-order balance between faster wheel-driven renewal and the loss of effective observation time predicted by the blocking approximation.

## Plot 3: Heatmap of Effective Sampled Mass Estimate

This heatmap presents the coupled effect of RPM and penetration depth at a representative bulk density of 0.5 g/cm^3. It is useful for identifying broad operating regions rather than a single one-dimensional trend.

Because both deterministic renewal count and sampled mass per refresh event contribute to the final estimate, the heatmap makes it easier to see where different RPM-depth combinations produce similar effective sampled mass estimate values.

## Plot 4: Blocked Time Fraction vs RPM

This plot separates the blocking model from the sampled-mass model. It shows how much of the acquisition is estimated to be unavailable because the spiderwheel fingers overlap the probe region under the binary blocking approximation.

The blocked fraction matters because it reduces effective observation time even in cases where wheel speed increases the rate of renewal opportunities.

## Plot 5: Sampled Mass per Refresh vs Penetration Depth

This plot shows the sampled mass per refresh event before the renewal-count multiplier is applied. It therefore isolates the sampled-volume approximation from the dynamic renewal approximation.

Reading Plot 5 together with Plot 1 helps distinguish two quantities that should not be conflated: msample is the per-refresh sampled mass, whereas meff,est is the effective sampled mass estimate accumulated over one acquisition.

## Plot 6: Contour Map of Effective Sampled Mass Estimate

This contour plot provides the same coupled RPM-depth view as the heatmap, but for the higher-density case of 0.6 g/cm^3. It is useful for visualizing smooth operating envelopes and approximate iso-mass contours.

The figure remains a first-order mechanistic map. It is useful for design sensitivity screening, but it should not be interpreted as a validated surface of statistically independent effective sample size.

## Results Interpretation

increasing bulk density increases the sampled mass per refresh event linearly in this simplified volume model and therefore increases the effective sampled mass estimate proportionally when the renewal count is unchanged

increasing penetration depth increases the assumed sampled volume linearly and therefore increases the per-refresh sampled mass linearly under the current approximation

increasing RPM decreases the characteristic crossing time and therefore increases the deterministic number of renewal opportunities, all else being equal

blocking reduces effective observation time and therefore reduces the effective sampled mass estimate relative to an unblocked case

these outputs are mechanistically useful for engineering comparison, but they do not by themselves establish statistical independence of refresh events or a validated independent effective sample size

## Limitations

partial blocking, partial transmission, and partial attenuation are not modeled; blocking is treated as binary

recirculation, revisit probability, and repeated presentation of the same powder increment are not modeled

particle-scale heterogeneity, segregation, and particle-count statistics are outside the present approximation

depth-dependent photon weighting is not represented; the sampling region is treated with a uniform first-order volume approximation

residence-time distributions and local mixing memory near the probe are not modeled

stochastic independence of refresh events is not modeled, so renewal count is not equivalent to validated independent sample count

the model does not yet convert effective sampled mass estimate into analytical variance, prediction uncertainty, or a validated statistical effective sample size

## Next Logical Extensions

replace binary blocking with a partial-blocking or partial-exposure model

include revisit probability or explicit recirculation of powder around the wheel

resolve the acquisition in time or phase so that exposure depends on wheel angle during the spectrum

introduce depth-dependent weighting instead of a uniform cylindrical volume approximation

estimate particle count as well as mass within the refreshed sampling region

apply a correlation correction between refresh events before interpreting the result as effective sample mass

link effective sampled mass estimate to analytical variance or prediction precision

## Export and Reporting

This export section writes timestamped CSV, PNG, JSON, and Word outputs into the setup-specific reports/ folder. python-docx and nbformat are required for Word export with Markdown rendering.

# Purpose

Setup C estimates a first-order effective sampled mass estimate for a spiderwheel presentation by combining sampled mass per refresh with a deterministic renewal count derived from wheel speed and blocked acquisition time.

# Assumptions

Uniform bulk density, cylindrical sampling volume, binary full-blocking simplification for wheel fingers, and deterministic refresh counting without revisit, recirculation, partial-blocking, or correlation corrections.

# Parameter Summary

Finger count: 4

Wheel radius (cm): 3

Probe radius (cm): 1.8

Finger width (cm): 0.3

Probe diameter (mm): 5

Acquisition time (s): 1

Density scenarios (g/cm^3): 0.4, 0.5, 0.6

RPM range: 10 to 35

Penetration depth range (mm): 0.1 to 2.0

# Exported CSV Files

results_full.csv

summary_table.csv

blocking_summary.csv

Detailed tables are saved separately as CSV files in the run folder.

# Figures

plot_01_estimate_vs_penetration_depth.png

plot_02_estimate_vs_rpm.png

plot_03_heatmap_density_05.png

plot_04_blocked_time_fraction_vs_rpm.png

plot_05_sampled_mass_per_refresh.png

plot_06_contour_density_06.png

# Closing Note

This report captures the notebook narrative, summary metadata, and exported artifacts for this run.