# Setup A Effective Sample Size Report

# Run Metadata

Setup name: setup_a

Run timestamp: 20260315_213602

Notebook filename: setup_A_static_effective_sample_size.ipynb

Run directory: C:\Users\Erik\Documents\Projects\EFS\reports\setup_a\20260315_213602

# Notebook Narrative

# Setup A - Effective Sample Size for Static NIR Reflectance Measurement

Static powder bed, circular probe window, deterministic first-generation model

## Background

Setup A represents a static powder bed measured with a contact or near-contact diffuse reflectance NIR probe. In this configuration, powder motion is neglected during acquisition, so the optical interaction zone can be approximated from probe geometry and effective penetration depth.

Effective sample size matters because it links measured spectral response to how much material actually contributes to the signal. This directly influences interpretation, robustness, and representativity.

A static powder bed is conceptually the simplest setup because dynamic transport and re-sampling effects are absent in the first approximation. The remaining challenge is optical interaction geometry.

It is important to distinguish the geometric interaction volume from the true optical sensitivity distribution. Real diffuse reflectance sampling is non-uniform with depth and lateral position. This notebook starts with a cylindrical approximation to provide a clear first-order model for sensitivity analysis and order-of-magnitude understanding.

## Objective

Simulate sampled volume and sampled mass for a 5 mm circular probe as a function of effective penetration depth from 0.1 mm to 2.0 mm, and examine the influence of powder bulk density.

## Theory

For a circular probe window:

First-generation sampled volume approximation:

Sampled mass approximation:

Symbols and units:

D_probe: probe window diameter [m]

A_probe: probe area [m^2]

d_pen: effective penetration depth [m]

rho_bulk: bulk density [kg/m^3]

V_sample: sampled volume [m^3]

m_sample: sampled mass [kg]

This is a first-generation geometric approximation. The true optical sensitivity distribution is not perfectly cylindrical.

## Assumptions

static powder bed

uniform bulk density

circular probe window

uniform effective penetration depth

cylindrical optical interaction volume approximation

no particle motion during acquisition

no resampling

no detailed radiative transfer model yet

## Input Parameters

Baseline and sweep configuration:

baseline probe diameter: 5.0 mm

penetration depth range: 0.1 mm to 2.0 mm (inclusive), step 0.1 mm

Setup A bulk densities (discrete scenarios): 0.40, 0.50, 0.60 g/cm^3

Setup A examines only these three representative bulk-density cases for moderately light to moderately dense pharmaceutical powder beds/blends. The intent is scenario comparison, not continuous interpolation.

Internal calculations use SI units.

## Interpretation of the Simulation Table

Each row corresponds to one penetration depth and one bulk-density scenario for static Setup A. Effective sample size is treated as sampled mass from a static illuminated volume and computed as:

where the illuminated area is fixed by probe geometry and penetration depth is swept from 0.1 to 2.0 mm.

## Plot 1: Sampled Volume vs Penetration Depth

This plot shows the geometric sampled volume as a function of penetration depth. In this model, sampled volume does not depend on bulk density.

## Plot 2: Effective Sample Size vs Light Penetration Depth

Setup A assumes a static powder bed. Effective sample size is approximated as bulk density x illuminated area x penetration depth, so the plotted quantity is the estimated sampled mass contributing to the reflectance measurement. Penetration depth is varied from 0.1 to 2.0 mm to examine sensitivity to optical sampling depth, and three bulk-density scenarios (0.4, 0.5, 0.6 g/cm^3) represent lighter to denser powder beds.

## Results Interpretation

effective sample size (sampled mass) scales linearly with penetration depth when illuminated area is fixed

effective sample size increases with bulk density at any fixed penetration depth

three density curves (0.40, 0.50, 0.60 g/cm^3) provide discrete scenario comparison for static Setup A

this remains a geometric first approximation rather than a full optical transport model

## Practical Implications

uncertainty in penetration depth strongly affects estimated effective sample size

bulk density matters when converting interaction volume to sampled mass

this framework is useful for comparing powders and probe setups

## Limitations

true photon sampling distribution is non-uniform

penetration depth is an effective parameter, not a sharp boundary

bulk density may vary with compaction and handling

particle-level sampling statistics are not yet included

no correction factor for optical weighting yet

## Next logical extensions

interaction volume correction factor

particle count estimation

sensitivity analysis for probe diameter

uncertainty bands for penetration depth

non-uniform optical weighting functions

## Export and Reporting

This export section writes timestamped CSV, PNG, JSON, and Word outputs into the setup-specific reports/ folder. python-docx and nbformat are required for Word export with Markdown rendering.

# Purpose

Setup A estimates static optical sampling mass for a fixed circular probe over a penetration-depth sweep and compares three representative bulk-density scenarios.

# Assumptions

Static powder bed, fixed illuminated area from probe geometry, uniform bulk density within each scenario, and a single effective penetration depth used as a first-order optical volume approximation.

# Parameter Summary

Probe diameter (mm): 5

Penetration depth range (mm): 0.1 to 2.0

Penetration depth step (mm): 0.1

Bulk densities (g/cm^3): 0.4, 0.5, 0.6

# Exported CSV Files

results_full.csv

summary_table.csv

input_summary.csv

Detailed tables are saved separately as CSV files in the run folder.

# Figures

plot_01_sampled_volume_vs_penetration_depth.png

plot_02_effective_sample_size_vs_penetration_depth.png

# Closing Note

This report captures the notebook narrative, summary metadata, and exported artifacts for this run.