# Setup B Effective Sample Size Report

# Run Metadata

Setup name: setup_b

Run timestamp: 20260315_213654

Notebook filename: setup_B_spoon_effective_sample_size.ipynb

Run directory: C:\Users\Erik\Documents\Projects\EFS\reports\setup_b\20260315_213654

# Notebook Narrative

# Setup B -- Effective Sample Size for Spoon-Based NIR Measurement

Independent spoon samples measured as repeated static optical measurements during one spectral acquisition

## Background

Setup B represents a spoon or flip-sampler interface where powder originates from a flowing process stream, is captured in a spoon, and is measured by the NIR probe while static during each spoon presentation.

The upstream process is dynamic, but each optical event is treated as a static-bed measurement during the spoon exposure. During one reported spectrum, multiple spoon presentations may occur. Under the assumptions used here, effective sample size is governed by optical interaction mass per spoon, total acquisition time, and spoon flipping frequency. It is not directly governed by upstream throughput in this first deterministic model.

## Objective

This notebook simulates Setup B effective sample size by quantifying:

static optical sampled mass per spoon

number of spoon samples contributing during one spectrum

effective sampled mass per spectrum

as functions of penetration depth, bulk density, flip frequency, and acquisition settings.

## Theory

For one spoon presentation (static measurement):

A_probe = pi\(\fracD_probe2\)^2

V_static \ approx A_probe\ d_pen

m_static = rho_bulk\ V_static

Acquisition timing:

t_acq = N_subscans\ t_int

t_spoon = \frac1f_flip

N_spoons = t_acq\ f_flip

Effective sampled mass during one reported spectrum:

m_eff = m_static\ N_spoons

Symbol definitions and units:

D_probe: probe diameter [m]

A_probe: probe area [m^2]

d_pen: effective penetration depth [m]

rho_bulk: bulk density [kg/m^3]

V_static: static sampled volume per spoon [m^3]

m_static: static sampled mass per spoon [kg]

N_subscans: number of averaged sub-scans [-]

t_int: integration time per sub-scan [s]

t_acq: total acquisition time [s]

f_flip: spoon flipping frequency [Hz]

t_spoon: spoon interval [s]

N_spoons: spoon samples contributing during one acquisition [-]

m_eff: effective sampled mass per spectrum [kg]

## Interpretation of the Model

In this setup, one spoon presentation is one independent static optical sample. If acquisition time spans multiple spoon presentations, the reported spectrum integrates multiple independent increments.

In this notebook, N_spoons is allowed to be non-integer as a deterministic first approximation for effective mass scaling. Under the stated assumptions, throughput is not explicitly modeled.

## Assumptions

spoon always completely filled

filling is reproducible

consecutive spoons are independent

powder is static during each spoon measurement

circular probe window

cylindrical optical interaction volume approximation

uniform bulk density

no resampling within a spoon

no segregation correction

no stochastic independence correction

no detailed radiative transfer model yet

## Input Parameters

Baseline optical, acquisition, and spoon-frequency settings are defined below. SI units are used internally, while engineering units are kept in labels and summary tables for readability.

## Interpretation of Simulation Table

Each row represents one parameter combination of penetration depth, bulk density, and time between spoon flips.

staticsampledmass_mg is the per-spoon static optical sampled mass.

spoons_contributing is the deterministic estimate of spoon presentations during one spectrum.

effectivesampledmass_mg is the scaled effective sampled mass per spectrum under independent spoon assumptions.

## Plot 1: Static Sampled Volume per Spoon vs Penetration Depth

This plot shows static sampled volume per spoon as a function of penetration depth for the cylindrical first-generation optical approximation.

## Plot 2: Static Sample Mass per Spoon (Representative Depth)

This plot shows static sample mass per spoon (mg) for bulk densities 0.4, 0.5, and 0.6 g/cm^3, using the Setup B spoon geometry at a representative penetration depth of 1.0 mm.

## Plot 3: Spoons Contributing per Spectrum vs Spoon Flip Interval

This plot shows how the number of spoon presentations contributing to one spectrum changes with time between spoon flips.

spoonsvsintervaldf = pd.DataFrame( { "spoonintervals": spoonintervalss, "spoonscontributing": [spoonscontributingfrominterval(acqtimes, float(s)) for s in spoonintervals_s], } )

figsetupbspoonsvsinterval, ax = plt.subplots(figsize=(7, 4.5)) ax.plot( spoonsvsintervaldf["spoonintervals"], spoonsvsintervaldf["spoonscontributing"], marker="o", ) ax.setxlabel("Time between spoon flips (s)") ax.setylabel("Spoons contributing per spectrum (-)") ax.settitle("Setup B: Spoons Contributing vs Spoon Flip Interval") ax.grid(True, alpha=0.3) figsetupbspoonsvsinterval.tight_layout() plt.show()

## Results Interpretation

Plot 2 shows static sample mass per spoon for three representative bulk densities: 0.4, 0.5, and 0.6 g/cm^3

Plot 4 uses only representative bulk density 0.5 g/cm^3

Plot 4 x-axis is time between spoon flips (s), not flip frequency (Hz)

shorter spoon intervals produce more independent spoon presentations during acquisition, increasing effective sample size

selecteddepthsmm = [0.2, 0.5, 1.0, 2.0] effectivemassvsintervaldf = resultsdf[ (resultsdf["bulkdensitygcm3"] == representativebulkdensitygpercm3) & (resultsdf["penetrationdepthmm"].isin(selecteddepths_mm)) ].copy()

figsetupbselecteddepths, ax = plt.subplots(figsize=(7, 4.5)) for penetrationdepthmm, groupdf in effectivemassvsintervaldf.groupby("penetrationdepthmm"): groupdf = groupdf.sortvalues("spoonintervals") ax.plot( groupdf["spoonintervals"], groupdf["effectivesampledmassmg"], marker="o", label=f"{penetrationdepth_mm:.1f} mm", )

ax.setxlabel("Time between spoon flips (s)") ax.setylabel("Effective sampled mass per spectrum (mg)") ax.set_title("Setup B: Effective Sampled Mass vs Spoon Flip Interval (0.5 g/cm^3)")

textstr = f"Total spectral acquisition time = {totalspectralacquisitiontimes:.2f} s" ax.text( 0.05, 0.95, textstr, transform=ax.transAxes, fontsize=10, verticalalignment="top", bbox=dict(boxstyle="round", facecolor="white", alpha=0.8), ) ax.legend(title="Penetration depth") ax.grid(True, alpha=0.3) figsetupbselecteddepths.tight_layout() plt.show()

## Limitations

independence between spoons is assumed

complete and reproducible filling is assumed

no explicit throughput effect is included

no dynamic filling physics is modeled

no partial spoon overlap model is included

no integer/discrete spoon capture model is enforced

no optical weighting correction yet

## Next logical extensions

enforce integer spoon counting or partial spoon weighting

include incomplete filling scenarios

include non-independent spoons

include spoon geometry explicitly

include particle count estimation

include uncertainty in penetration depth and bulk density

## Export and Reporting

This export section writes timestamped CSV, PNG, JSON, and Word outputs into the setup-specific reports/ folder. python-docx and nbformat are required for Word export with Markdown rendering.

# Purpose

Setup B estimates effective sampled mass per spectrum by combining static optical mass per spoon with the deterministic number of spoon presentations during one spectral acquisition.

# Assumptions

Each spoon presentation is treated as an independent static optical event, spoon filling is reproducible, and effective sample size scales with flip frequency over the acquisition window.

# Parameter Summary

Probe diameter (mm): 5

Penetration depth range (mm): 0.1 to 2.0

Penetration depth step (mm): 0.1

Bulk densities (g/cm^3): 0.4, 0.5, 0.6

Subscans: 256

Integration time (ms): 15

Acquisition time (s): 3.84

Spoon intervals (s): 1, 2, 3, 4, 5

# Exported CSV Files

results_full.csv

summary_table.csv

input_summary.csv

flip_summary.csv

plot2_static_sample_mass_table.csv

plot4_spoon_interval_effect_table.csv

selected_depths_interval_table.csv

Detailed tables are saved separately as CSV files in the run folder.

# Figures

plot_01_static_sampled_volume_per_spoon.png

plot_02_static_sample_mass_per_spoon.png

plot_04_effective_sample_size_vs_flip_interval.png

# Closing Note

This report captures the notebook narrative, summary metadata, and exported artifacts for this run.