# ESS Comparison Across Setups A-E

# Comparison of Effective Sample Size Across Setups A-E

This notebook compares the effective sample size (ESS) across Setups A-E under one aligned operating basis.

The main output is one comparison table covering:

Setup A at 5 mm probe diameter and 1.8 s acquisition time

Setup B at 5 mm probe diameter, 95 kg/h throughput metadata, 3.6 s acquisition time, and 3 s spoon interval

Setup C at 5 mm probe diameter, 95 kg/h throughput, 1.8 s acquisition time, and 32 RPM

Setup D at 5 mm probe diameter, 95 kg/h throughput, and 1.8 s acquisition time

Setup E at 95 kg/h throughput, 1.8 s acquisition time, and probe diameters of 5 mm and 25 mm

The 1.8 s spoon scenario for Setup B is intentionally omitted.

## Shared Assumptions

Bulk density is fixed at 0.4 g/cm^3 for every comparison row.

Penetration depth is aligned to 1.0 mm for every setup because a common optical depth is required for direct comparison and 1.0 mm is already used as a representative row in the existing setup reports.

Setup A is static, so acquisition time is reported for alignment only and does not change the ESS value.

Setup B keeps 95 kg/h as aligned metadata, but the existing deterministic spoon model does not use throughput directly.

Setup E uses the swept-capsule unique-exposure ESS proxy from the PAT-rig notebook.

The comparison table is the primary notebook output.

## Calculation Basis

Setups A, B, and E are calculated directly from their notebook models with the aligned parameters above. Setups C and D are pulled from the latest exported report tables at the exact aligned operating point, so the comparison notebook reuses the established implementation instead of recreating the full mechanistic code path.

## Main Comparison Table

The table below is the primary notebook output.

## Short Interpretation

Setup E with the 25 mm spot is highest because the large instantaneous footprint dominates the total unique-exposure mass.

Setup A is lowest because the model is only a static illuminated volume with no renewal term.

Setup C is above Setup D because 32 RPM generates more refresh events in 1.8 s than the pipe-flow case at 95 kg/h, even after blocking is removed.

Setup E drops sharply when the spot shrinks from 25 mm to 5 mm because both the circular footprint and the swept increment contract.

Setup B at 3.6 s only slightly exceeds Setup A because 3 s spoon flips yield just 1.2 spoon presentations during one spectrum.

## Export and Reporting

This section writes a timestamped comparison folder under reports/setup_comparison/, saves the aligned scenario table and the final comparison table as CSV, writes metadata as JSON, and generates a Word document with the notebook markdown rendered together with the comparison table.

# Comparison Table

| Setup | Scenario | Basis | Probe mm | Depth mm | Throughput kg/h | Acq s | Spoon s | RPM | Density g/cm^3 | ESS mg | Rank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | A1 | Static illuminated volume | 5 | 1.000 |  | 1.800 |  |  | 0.400 | 7.854 | 6 |
| B | B2 | Static mass times spoon presentations | 5 | 1.000 | 95.000 | 3.600 | 3.000 |  | 0.400 | 9.425 | 5 |
| C | C1 | Blocked deterministic renewal | 5 | 1.000 | 95.000 | 1.800 |  | 32.000 | 0.400 | 120.281 | 2 |
| D | D1 | Flow-refresh proxy | 5 | 1.000 | 95.000 | 1.800 |  |  | 0.400 | 56.213 | 3 |
| E | E1 | Swept capsule unique exposure | 5 | 1.000 | 95.000 | 1.800 |  |  | 0.400 | 38.093 | 4 |
| E | E2 | Swept capsule unique exposure | 25 | 1.000 | 95.000 | 1.800 |  |  | 0.400 | 347.547 | 1 |